Pressure Detection Project

Mastering Embedded System Online Diploma

www.learn-in-depth.com

First Term Project ( Final Project 1 )

Eng. Ahmed Nabil Mahmoud

https://www.learn-in-depth.com/online-diploma/ahmed.nabil.9711%40gmail.com

Contents

1. case study	2

                                                        Requirements	2

	Assumptions	2

	 Versioning	2

2. Method	3

Software developing life cycle & software testing life cycle	3

3. Requirement Diagram	4

4. Space Exploration	5

	Features	5

5. System Analysis	6

	Use Case Diagram	6

	Activity Diagram	7

	Sequence Diagram	8

6. System Design	9

	Block Diagram	9

	State Machine For Pressure_Sensor	10

	State Machine For Main Algorithm	11

	State Machine For Alarm_Control	12

	State Machine For Alarm_Actuator	13

7. C codes	14

	Pressure_sensor_.c/.h	14

	Main_algorithm_.c/.h	16

	Alarm_control_.c/.h	18

	Alarm_actuator_.c/.h	20

	States.h	22

	App.c	23

8. Building_Files	24

	startup.c	24

	linker_script	25

	makefile	26

9. Software_analysis	27

	.map file	27

	Symbols table	28

	Sections table	29

10. Simulation	30

	Pressure less than threshold	30

	Pressure more than threshold	31

# 1. case study

## Requirements

1. Pressure detector that informs the cabin crew when pressure exceeds a threshold of 20 Bar.

2. The system informing is acting by a Led-alarm which should last for 60 sec.

3. The system should keep tracking for pressure values.

## Assumptions

1. No setup or shut down for micro-controller.

2. No maintenance for micro-controller.

3. Pressure sensor provides accurate readings.

4. Both sensor & actuator never fail.

5. No power constrains

## Versioning

The probability of adding a feature to store pressure values versus time in a flash memory in the next version.

# 2. Method

## Software developing life cycle & software testing life cycle

The ( SDLC ) & ( STLC ) will be approached according to the V-Model.

# 3. Requirement Diagram

# 4. Space Exploration

Micro-controller : stm32f103c8t6 SoC as it meets all technical requirments needed for this project as it is marked by :  small size and contains acceptable flash memory as well as being cost efficient and have a suitable processor which is : ARM Cortex-M3 32bit with 72 MHz operating frequency.

## Features

operating voltage range -> 2 : 3.6 V.

64Kbytes Flash memory.

20KbytesSRam.

CRC calaculation unit , 96bit unique id.

Two 12bit, 1µs A/D converter (up to 10 channels).

7 channel DMA controller , 3 genral purpose timer & 1 advanced

Controller timer.

37 fast I/O ports.

Serial wire debug (SWD) & JTAG Interfaces .

Two SPI , two I2C , three USART , one USB & one CAN interfaces.

Ambient operating temperature range from -40˚C to 85˚C.

# 5. System Analysis

## Use Case Diagram

## Activity Diagram

## Sequence Diagram

# 6. System Design

## Block Diagram

## State Machine For Pressure_Sensor

## State Machine For Main Algorithm

## State Machine For Alarm_Control

## State Machine For Alarm_Actuator

# 7. C codes

## Pressure_sensor_.c/.h

.h file

.c file

## Main_algorithm_.c/.h

.h file

.c file

## Alarm_control_.c/.h

.h file

.c file

## Alarm_actuator_.c/.h

.h file

.c file

## States.h

## App.c

# 8. Building_Files

## startup.c

## linker_script

## makefile

# 9. Software_analysis

## .map file

## Symbols table

## Sections table

# 10. Simulation

## Pressure less than threshold

## Pressure more than threshold